Documentation on Computer Architecture CPU and Cores

CACHE

RAM

Introduction

Computer Architecture describes the design and structure of a computer system — how hardware components (processor, memory, I/O, buses, etc.) are arranged and how they interact, and also how software is made to run on that hardware.

Typically, we split it into two views:

1. Instruction Set Architecture (ISA): defines how software talks to hardware — what instructions are supported, how data types are handled, registers, addressing modes, etc.

2. Microarchitecture / Hardware System Architecture: describes how those instructions are implemented in actual hardware — CPU datapaths, control logic, memory hierarchy, I/O systems, buses.

CPU (Central Processing Unit): The “brain” — carries out instructions. Internally consists of:

ALU (Arithmetic Logic Unit): performs arithmetic (add/sub/mult/div) and logical operations (AND, OR, comparisons, bit shifts, etc.) on data.

Control Unit (CU): manages the execution — fetches instructions, decodes them, controls data flow, coordinates with memory and I/O, etc.

Registers: Very fast small storage inside CPU for temporary data, addresses, instructions. Example registers: Program Counter (PC), Instruction Register (IR), Memory Address Register (MAR), Memory Data Register (MDR), Accumulator or general-purpose registers.

Memory / Storage Subsystem: Where instructions and data reside. This includes multiple levels (hierarchy) depending on speed & size.

Input / Output (I/O) Units: Interfaces to get data in (keyboard, mouse, network, sensors, etc.) and output data (display, disk, network, actuators, etc.).

Interconnection / Buses / Communication Paths: Buses carry data, addresses, and control signals among CPU, memory, I/O units. They tie together the components and enable communication.

Instruction Execution: How Programs Run

When you run a program, the CPU follows the fetch–decode–execute cycle (also called instruction cycle). First it fetches the next instruction from memory, decodes what needs to be done, then executes: fetch operands (if needed), execute operation (via ALU), store result.

Registers (inside CPU) store the instruction, addresses, data, and intermediate results — this is much faster than reading/writing from main memory each time.

Memory Hierarchy — Speed vs Size vs Cost Tradeoff

A central concept in computer architecture is the memory hierarchy: storing data/instructions at multiple levels that trade off speed, size, and cost.

Hierarchy (from fastest/smallest → slowest/largest):

| Level | Description |
| --- | --- |
| Registers | Inside CPU, fastest access, very limited size. Used for immediate data and intermediate results. |
| Cache Memory | A small, very fast memory between CPU and main memory. Holds frequently used data/instructions so CPU doesn’t always need to go to RAM. Often implemented using SRAM for speed. |
| Main Memory (RAM) | Larger memory used to hold currently active data and instructions, slower than cache but much larger capacity. |
| Secondary / Persistent Storage (Disk, SSD, external memory) | Non-volatile storage for long-term data, programs, files. Slowest access but largest capacity. |

Why this hierarchy? Because CPU is very fast; if it had to wait for data/instructions from slow storage (disk or even RAM) every time — efficiency would be terrible. Memory hierarchy ensures that most of the time CPU works with fast memory (registers/cache), reducing wait time.

CPU Cache in Detail

- CPU cache is a dedicated small, fast memory near the CPU that stores copies of frequently accessed data/instructions from main memory.

- Cache memory reduces average access time because accessing cache is much faster than accessing main RAM.

- Most modern CPUs use multilevel caches: e.g. L1 (closest and fastest, often split into instruction and data caches), L2 (larger but slower than L1), and often L3 (shared among cores).

- Design decisions about cache size, level hierarchy, replacement policies, and shared vs separate caches significantly affect performance, cost, and power consumption.

.

What is RAM — Basic Definition & Role

- RAM is the computer’s main memory (also called “primary memory” or “main memory / system memory”) used to store data and instructions that the CPU needs right now (or soon) for active tasks.

- The “random access” in RAM means the CPU can read or write data at any memory cell location directly, without needing to traverse memory sequentially — which makes access fast and efficient.

- RAM is volatile memory: when the computer powers off or restarts, data stored in RAM is lost.

- Purpose: When you run programs (OS, applications, background tasks), they and their data are loaded into RAM so CPU can work on them quickly; RAM serves as a bridge between fast CPU and slower permanent storage (like SSD/HDD).

How RAM Works & Memory Hierarchy

- Computers typically use a memory hierarchy: small/fast (registers, cache), medium (RAM), and large/slower storage (SSD/HDD). This hierarchy balances speed, capacity, cost, and complexity.

- CPU first tries to use fastest memory (registers or cache), if data isn’t there it goes to RAM, and if not even there, to disk/storage. RAM is thus a crucial middle-layer — fast enough for active data, large enough to hold applications and working data.

- Because RAM is faster than disk storage, keeping active program code and data in RAM significantly improves performance; if there is too little RAM, system may rely heavily on slower storage, causing lag.

## Types of RAM & Their Differences

RAM itself comes in a few variants. The two fundamental categories:

Static RAM (SRAM)

- Uses flip-flop circuits (multiple transistors per bit) to store each bit.

- Does not require refreshing to retain data (just needs power supply).

- Very fast access times, low latency — much faster than DRAM.

- But: more expensive per bit, takes more physical space (less dense), and typically lower capacity.

- Common Use: Usually used for cache memory (inside CPU or close to CPU), where speed is critical and small size suffices.

Dynamic RAM (DRAM)

- Stores each bit as a charge in a tiny capacitor (with a transistor to read/write).

- Because capacitors leak charge, DRAM must be periodically refreshed (many times per second) to maintain data.

- Slower access compared to SRAM (higher latency) due to capacitor nature and refresh overhead.

- But: more dense (more bits per chip), cheaper per bit — allowing large memory capacities (e.g. gigabytes) at reasonable cost.

- Common Use: DRAM is the standard for main system memory (RAM) in desktops, laptops, servers.

Other modern variants/types:

- Synchronous DRAM (SDRAM): A kind of DRAM whose operation is synchronized with the system clock, which helps increase performance.

- DDR (Double Data Rate) SDRAM: A widely used form where data is transferred on both rising and falling edges of clock signal, increasing effective data transfer rate. (DDR2, DDR3, DDR4, DDR5 are successive generations improving speed, efficiency)